INSTRUCTION MANUAL

SMART SHOE RACK

Presented by

CHONG KHENG CHEN

SPECIFICATIONS

- Power supply: 12v

- Control system: Esp 32 microcontroller with Blynk App.

- Output data: stepper motor.

- Capacity shoe: 9 pair of shoes

ELECTRIC COMPONENT

- Esp32 -1 uint

- Stepper motor -3 unit

- Limit switch-3 unit

- Relay -2 unit

- Barcode scanner- 1 unit

- Ir sensor – 1 unit

- Pilot lamp-1 unit

- Push start button- 1 unit

- Switch on/off -1 unit

- limit switch -3 unit

SAFETY PRECAUTIONS

Electrical safety: Use only the adapter.

Input 100-240v ~50/60Hz 2A.

Output :12v ~2500mA

Children supervision: Keep this application out of reach of children.

Moisture Protection: Do not place the device near water or any other liquid.

Avoid putting your hands where the motor.

PART IDENTIFICATION

- Main body: shoe rack with 3D rail.

- Control Panel: Includes on/off switch.

- ESP32: Located inside the control compartment.

- Ir sensor: Detects have shoe box at home position or not.

- Barcode scanner: Scan the barcode data.

- Stepper motor: Take the shoe from the home position to the specified slot.

- Power supply: Provides the correct voltage and current for a device.

- Limit switch: To find 0,0,0 position.

- Pilot lamp: When the ir sensor have detect the pilot lamp will on.

- Switch: To switch on / off.

- Push start button: To give the motor back to home position (0,0,0).

OPERATING INSTRUCTIONS

Before using:

- Make sure the place is safety and stable surface.

- Ensure stable power supply.

- Make sure the wire is connected correctly.

- Make sure the input is AC / DC.

Step-by-step guide:

- Power on: Plug in the adapter into 100-240v outlet.

- Home: Pless the push start button to let motor back to home position (0,0,0).

- Ir sensor: If the ir sensor is detect the shoe box the pilot lamp will on.

- Barcode scanner: Pless the button of barcode scanner at blynk app .

- Motor process: The motor will take the shoe box to the specify slot.

- Back to home position: After the motor put the shoe box at the specified slot the motor will be back to home position (0,0,0).

- Led at Blynk: When the motor back to home position the led at Blynk App will be on.

- Take the shoe from the slot: Press the button at Blynk App after that the motor will go to the slot and take the shoe box back to home position (0,0,0).

- Completion: Once the process is complete, a notification will be sent to your smartphone.

BLYNK INTERFACE FUNCTIONS

Led slot 1/2: when the slot has shoe box the led will on while no have shoe box the led will off.

Button slot 1/2: we press the button the motor will go to the slot and take the shoe box to home position.

Button barcode scanner: When we press the button the barcode scanner will scan the barcode data.

CLEANING OF MAINTENANCE

- Unplug the unit before cleaning.

- Use a damp cloth to wipe the exterior of the unit.

- Do not immerse the unit in water or any other liquid.

- Check the 3D rail regularly to avoid jamming.

TROUBLESHOOTING

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| No power to the unit. | Not plugged in properly. | Check the power plug and socket. |
| No notification received. | Wi-Fi connection issue. | Check the Wi-Fi connection and restart the app. |
| Barcode scanner unfunctional. | Barcode data no have in the list. | Add on the barcode data in the list. |
| Push start button unfunctional. | The wire of it is issue. | Change the new wire. |